## 4.1 Hydrological Analysis:

Hydrological Design

In hydrological analysis catchment area was calculated from topo map.

To calculate design discharge, we apply various methods, and among them, suitable discharge was chosen. In all methods, we choose Return Period: 100 Years

Outlet point @ 28 19 N 18 54 E

The design discharge can be calculated by using various methods as given below:

Design Parameters:

Catchment Area = A = 105.7 km2

Area under 3000m = 78,981,907m2

Perimeter under 3000 = 41970m

Rational Formula

Figure 1 Catchment Area

Q = CIA/360

Where,

tc=0.01947L0.77S-0.385

L = 21.6 km

S = 4398/21600

= 0.2042

Tc= 78.08 minutes

C = 0.2

itc,p= KTxtc+an

Where, K=5.914,

T= 100 years

x= 0.1623

a= 0.50

n= 1.0127

i= 68.8 mm/hr

Q = CiA/360

= 404.01 m3/sec (Taking C = 0.2 for hilly forest)

Figure 2 Snow covered Catchment area

Figure 2 Catchment area under 3000m

Dickens Formula (1865):

QP = CDA3/4

Where Cd = 14

A = 105.7km2

Q = 461.51 m3/s

Modified Dickens:

Q = CTA3/4

Where Ct=2.342log100.6T×log101185P+4

T = 100

As = Snow covered catchment area = 1.90km2

P=100×As+6A = 1.854

Ct = 15.68

Q = 516.89m3/s

Ryve’s formula:

Q = CRA2/3

Figure 3 Catchment area under 3000

Where CR = 10.2

Q = 228.0255m3/s

WECS Method:

The formula for 100-year return period is given by
            Q100=14.63(A3000+1)0.7342

Where, A3000 = 78.98 km2

Q100 = 365.1m3/s

Figure 4 Contour map overlay and tracing of catchment area under 3000

Figure 5 River elevation profile

Figure 6 Ridge lines

Figure 7 Contour map (Interval 1000m)

| S.No | Method | Discharge |
| --- | --- | --- |
| 1 | Dicken’s Formula | 461.51 m3/s |
| 2 | Ryves Formula | 228.025 m3/sec |
| 3 | WECS | 365.1 m3/sec |
| 4 | Rational method | 404.01 m3/sec |
| 5 | Modified Dicken’s Formula | 516.89 m3/s |

Table: Discharge Table